Appendix N1

Technical Requirements

Product Name

- Ferric chloride (FeCl3) (40%)

Specifications

TYPE -   2

Purity    (FeCl,)                   - %40+ 2 (m/m)

Appearance                        - Dark    Brown   liquid

Density (20°C)                    - 1,435 ± 0,05 (gr/cm3)

Iron        (II)                          - % 2,5(m/m)/Fe(III)

APPLICATION    FIELDS:

Potable and industrial water treatment

Special requirements for shipment - Compulsory

- Tanker

Should be transported by 4 division tankers.

Tanker Material: should be covered with polyester.

Tanker should be usable for storing and transporting the Ferric 3 Chloride

Tank capacity is usually minimum 25 tons (17m3). We need to transport 17 tons in each tanker.

To unload the tanker special pump is necessary as well as a 20-meter-long pipe and a special connector - DN 80 mm, with the help of which Ferric 3 Chloride will be loaded to our reservoir.

Transportation and unloading procedures should be arranged by the seller, all above mentioned fittings should be provided by the Seller as well.

Please find the photos of the Tanker and the reservoir bellow

Our Reservoir in Gardabani:

Our reservoir is 5-6-meter-high but the discharging point is at 1-meter height approximately, the point of discharge is colored in the above photo.

The tanker will not be able to go inside the building, so it is of vital importance that the Seller had minimum 20-meter-long pipe for unloading the Ferric Chloride.

You can see from the photo the discharging point is at the reservoir pipe which runs upwards, that’s why it is absolutely necessary that the tanker had a special pump to discharge the liquid in the reservoir. This pump should be specially designed for Ferric Chloride and/or other strong chemicals.

You can also see from the photo that there is a necessity of a special connector and this special connector should be DN 80 mm. Without the proper connector it will be impossible to discharge the Ferric Chloride safely into the reservoir.

Tanker example photos:

Pump and pipe examle photos:

Discharge point close-up photo: