General Technical Assignment of the Bodorna Due Dilligence: Dam Field-Instrumental, Engineering Research and Monitoring System

# Recognosing the place and finding technical documentation

- Research works.

- Design of the monitoring system.

- Tool Installation

- BOQ elaboration

- Term of execution

- Field studies:

- Ortho-topographical capture/planning of the full perimeter of the dam and the lower bay.

- Geophysical survey:

  - Detailed geoelectric tomography along the dam trunk and base to detect filtration, possible sufosive phenomena and anomalies;

  - Perhaps other methods may be used (GPR, Electro-magnetic, Seismic....)

- Geotechnical drilling/research and arrangement of piesometers (preferably according to the initial results of geophysical research...)

- Taking water samples from the reservoir and filtering sources and analyzing them in a laboratory to detect suffusion processes and more...

- A one-time campaign to identify/measure the costs of surface filtering sources (this infobox will also be useful for determining and arranging the parameters of permanent expenditure nodes)...  At the same time, one-time measurements of all inflow-outflow costs of all other inflows and drain costs can be carried out and the number of filters can be determined ....

- Determining the volume of work

- Engineering Studies:

- Analysis/evaluation of dam sustainability and filtration regimes (baseline) based on the results of the field study and factual data, the issuance of appropriate recombinations, including the development of a monitoring system  and observation campaign concept.

- Dam breakthrough modeling

- Determining the volume of work

Monitoring System Design

- Monitoring system:

- Piezometers (arranged as part of the geotechnical survey)

- Vertical and horizontal displacement of dams and concrete structures - geodetic network (arrangement of pumps and monumnets (stamps) on dam snow...

- Arrangement of permanent automatic maintenance nodes on filtered sources

- Determining the volume of work

Compilation of total cost estimates.