Technical Data

- Jetting & Vacuum truck (Sewer cleaning industrial vehicle)

1 unit

- Chassis

- Axle: 6x4

- suspension: Air spring suspension

- Tank made of carbon steel, cylindrical section, with dished ends

  - Total capacity: 12-18 m3; ideally 12m3 or within the given range.

  - Tank must be divided in two parts:

    - water tank: 6-10 m3

    - Sludge tank: 6-8 m3

The front compartment should be intended for the transport of clean water and should include:

- one 500mm diameter manhole with a quick opening, allowing access to the interior for cleaning and maintenance.

- Water loading installation through carbon steel hydrants on the right side (preferable).

- The level indicator and overflow pipe with an outlet at the bottom of the compartment.

The rear compartment is intended for the transport of sludge and includes the following elements:

- One 500mm diameter manhole with pressure closure using manual butterfly locks.

- Rear compartment with fully opening dished end, hydraulically operated.

  - 4-inch (10.16cm) high-pressure suction hose with pneumatic valve

  - 6-inch (15.24cm) outlet hose with pneumatic valve

- Vacuum system

  - type: rotoral vacuum pump with air cooling system

  - flow rate: > 1500 m3/h

  - maximum vacuum: 90 or more

  - Sludge pumping depth: > 10 m

  - sludge pumper must also pump dry air

- Suction hose and hydraulic bearings

  - Diameter : 120 mm

  - Triple side operating system (3000)

  - A boom located on the top of the tank, rotatable, elevatable, and extendable;

- High pressure line

  - Power: > 100 Kwt

  - Max. Flow Rate: > 346 l/min

  - Max. Pressure: 250 bar

- High pressure main hose

  - 1800 operating

  - Length: 100-120 metre

  - Diameter: 2.54 – 3.81 cm (1-11/2 inch)

  - Manual operating hose: 50-60m.  Diameter: 1.27 cm (1/2 inch)

- Accesories

- Nozzles

| Photo | Description | QTY | Hose con. | Minimum /GPM | Pipe Diameter | Dimensions | Weight | Rear Jets | Front Jets | Radial Jets | Tier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pipe wolf nozzle 3D | 2PCS | 1 ¼ “ | 80 | 8”-24” | 7.48”X3.74” | 11.9lbs. | 6 | 3 |  | 3 |
|  | Pipe wolf nozzle 3D | 2PCS | 1” | 60 | 6”-15” | 6.29”X2.83” | 5.73lbs. | 6 | 3 |  | 3 |
|  | CHISEL NOZZLE | 2PCS | 1” | 60 | 8”-20” | 6.29”X2.95” | 5.51lbs. | 6 | 5 | ___ _ | 2 |
|  | CHISEL NOZZLE | 2PCS | 1” | 80 | 8”-15” | 3.74”X2.36” | 1.50lbs. | 8 | 4 | ___ _ | 2 |
|  | RADIAL CHISEL NOZZLE | 2PCS | 1” | 80 | 8”-20” | 6.29”X2.95” | 5.51lbs. | 6 | 5 | 6 | 2 |
|  | RADIAL CHISEL NOZZLE | 2PCS | ¾” | 30 | 6”-12” | 4.13”X2.36” | 2.42lbs. | 6 | 5 | 6 | 2 |

- Reducer & Collar

| ENLARGING COLLAR – STAINLESS STEEL |  |  |  |
| --- | --- | --- | --- |
|  |  |  | Quantity |
| ¾” male | → | 1” female | 2 PCS |

| REDUCER – STAINLESS STEEL |  |  |  |
| --- | --- | --- | --- |
|  |  |  | Quantity |
| 1 ¼ ” male | → | 1” female | 2 PCS |

- Special Demands

- special coaching and training to be performed by Coach certified by manufacturing factories.

- a supplier must have 30-year experience of manufacturing similar/analogous equipment.

- Preferred Demands

- Warranty should cover vehicle and jetting & vacuum unit across of Georgia (in case of other country, every expanse including transportation and lead time must be covered by a supplier).

- Soundproofing of the equipment.

- Wireless remote control;

- Jetting & Vacuum truck (Sewer cleaning industrial vehicle)

1 unit

- Chassis

- Axle: 6x2

- Suspension:  Air spring suspension

- Tank made of carbon steel, cylindrical section, with dished ends.

- tank capacity: 12-18 m3; ideally 12m3 or within the given range.

  - tank must be divided in two parts:

    - water tank: 6-10 m3

    - Sludge tank: 6-8 m3

The front compartment should be intended for the transport of clean water and should include:

- one 500mm diameter manhole with a quick opening, allowing access to the interior for cleaning and maintenance.

- Water loading installation through carbon steel hydrants on the right side (preferable).

- The level indicator and overflow pipe with an outlet at the bottom of the compartment.

The rear compartment is intended for the transport of sludge and includes the following elements:

- One 500mm diameter manhole with pressure closure using manual butterfly locks.

- Rear compartment with fully opening dished end, hydraulically operated.

  - 4-inch (10.16cm) high-pressure suction hose with pneumatic valve

  - 6-inch (15.24cm) outlet hose with pneumatic valve

- Vacuum System

  - type: rotoral vacuum pump with air cooling system

  - flow rate: > 1500 m3/h

  - maximum vacuum 90 and more

  - sludge pumping depth: > 10m

  - sludge pump must also pump dry air

- Suction hose and hydraulic bearings

  - diamterer: 120 mm.

  - Triple side operating system (3000)

  - A boom located on the top of the tank, rotatable, elevatable, and extendable;

- High-pressure line

  - power: > 100 kwt

  - maximum flow rate: > 346 l/min

  - maximum pressure: 250 bar

- High-pressure main hose

  - 1800  operating

  - length: 100-120 meter

  - diameter: 2.54 – 3.81 cm. (1-11/2 inch)

  - manual operating hose: 50-60 meter, dimater: 1.27cm  (1/2 inch)

7.	Accesories:

- Nozzles

| Photo | Description | QTY | Hose con. | Minimum /GPM | Pipe Diameter | Dimensions | Weight | Rear Jets | Front Jets | Radial Jets | Tier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pipe wolf nozzle 3D | 2PCS | 1 ¼ “ | 80 | 8”-24” | 7.48”X3.74” | 11.9lbs. | 6 | 3 |  | 3 |
|  | Pipe wolf nozzle 3D | 2PCS | 1” | 60 | 6”-15” | 6.29”X2.83” | 5.73lbs. | 6 | 3 |  | 3 |
|  | CHISEL NOZZLE | 2PCS | 1” | 60 | 8”-20” | 6.29”X2.95” | 5.51lbs. | 6 | 5 | ___ _ | 2 |
|  | CHISEL NOZZLE | 2PCS | 1” | 80 | 8”-15” | 3.74”X2.36” | 1.50lbs. | 8 | 4 | ___ _ | 2 |
|  | RADIAL CHISEL NOZZLE | 2PCS | 1” | 80 | 8”-20” | 6.29”X2.95” | 5.51lbs. | 6 | 5 | 6 | 2 |
|  | RADIAL CHISEL NOZZLE | 2PCS | ¾” | 30 | 6”-12” | 4.13”X2.36” | 2.42lbs. | 6 | 5 | 6 | 2 |

- Reducer and collar

| ENLARGING COLLAR – STAINLESS STEEL |  |  |  |
| --- | --- | --- | --- |
|  |  |  | Quantity |
| ¾” male | → | 1” female | 2 PCS |

| REDUCER – STAINLESS STEEL |  |  |  |
| --- | --- | --- | --- |
|  |  |  | Quantity |
| 1 ¼ ” male | → | 1” female | - PCS |

8.	Special demands

- special coaching and training to be performed by Coach certified by manufacture factories.

- a supplier must have 30 years experience of manufacturing similar/analogous equipment.

- Preferred demands

- Warranty should cover vehicle and jetting & vacuum unit across of Georgia (in case of other country, every expanse including transportation and lead time must be covered by a supplier).

- Soundproofing of the equipment.

- Wireless remote control;

- Jetting & Vacuum small truck (Sewer cleaning industrial vehicle)

1 unit

- Chassis:

- Axle: 4x4 wheels

- suspension: air spring suspension

- Tank made of carbon steel, cylindrical section, with dished ends

  - Total capacity: 7-11 m3; ideally 9m3 or within the given range.

  - Tank must be divided in two parts:

    - water tank: 3-5 m3

    - sudge tank: 4-6 m3

The front compartment should be intended for the transport of clean water and should include:

- one 500mm diameter manhole with a quick opening, allowing access to the interior for cleaning and maintenance.

- Water loading installation through carbon steel hydrants on the right side (preferable).

- The level indicator and overflow pipe with an outlet at the bottom of the compartment.

The rear compartment is intended for the transport of sludge and includes the following elements:

- One 500mm diameter manhole with pressure closure using manual butterfly locks.

- Rear compartment with fully opening dished end, hydraulically operated.

  - 4-inch (10.16cm) high-pressure suction hose with pneumatic valve

  - 6-inch (15.24cm) outlet hose with pneumatic valve

- Vacuum system:

  - type: rotoral vacuum pump with air cooling system

  - flow rate: > 1500 m3/h

  - maximum vacuum: > 90

  - sludge pumping depth: > 10m

  - sludge pumper must also pump dry air

- Vacuum hose and hydraulic bearings:

  - diameter: 120mm

  - triple-side operating system (3000)

  - A boom located on the top of the tank, rotatable, elevatable, and extendable;

- high-pressure line

  - power: > 80 kwt

  - maximum flow rate: > 280-300 l/min

  - maximum pressure: 250 bar

- high-pressure main hose

  - 1800 operating

  - length: 100-120m

  - diameter: 1.9-2.54cm (3/4-1 inch)

  - manual operating hose: 50-60m; diameter: 1.27cm (1/2 inch)

- accessories:

- Nozzles

| Photo | Description | QTY | Hose con. | Minimum /GPM | Pipe Diameter | Dimensions | Weight | Rear Jets | Front Jets | Radial Jets | Tier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pipe wolf nozzle 3D | 2PCS | 1 ¼ “ | 80 | 8”-24” | 7.48”X3.74” | 11.9lbs. | 6 | 3 |  | 3 |
|  | Pipe wolf nozzle 3D | 2PCS | 1” | 60 | 6”-15” | 6.29”X2.83” | 5.73lbs. | 6 | 3 |  | 3 |
|  | CHISEL NOZZLE | 2PCS | 1” | 60 | 8”-20” | 6.29”X2.95” | 5.51lbs. | 6 | 5 | ___ _ | 2 |
|  | CHISEL NOZZLE | 2PCS | 1” | 80 | 8”-15” | 3.74”X2.36” | 1.50lbs. | 8 | 4 | ___ _ | 2 |
|  | RADIAL CHISEL NOZZLE | 2PCS | 1” | 80 | 8”-20” | 6.29”X2.95” | 5.51lbs. | 6 | 5 | 6 | 2 |
|  | RADIAL CHISEL NOZZLE | 2PCS | ¾” | 30 | 6”-12” | 4.13”X2.36” | 2.42lbs. | 6 | 5 | 6 | 2 |

- Reducer and collar

| ENLARGING COLLAR – STAINLESS STEEL |  |  |  |
| --- | --- | --- | --- |
|  |  |  | Quantity |
| ¾” male | → | 1” female | 2 PCS |

| REDUCER – STAINLESS STEEL |  |  |  |
| --- | --- | --- | --- |
|  |  |  | Quantity |
| 1 ¼ ” male | → | 1” female | 2 PCS |

- Special Demands

- special coaching and training to be performed by Coach certified by manufacture factories.

- a supplier must have 30-year experience of manufacturing similar/analogous equipment.

- Preferred demands

- Warranty should cover vehicle and jetting & vacuum unit across of Georgia (in case of other country, every expanse including transportation and lead time must be covered by a supplier).

- Soundproofing of the equipment.

- Wireless remote control;