Technical requirements for an elaboration of PLC code and HMI screens for Project.

Tbilisi 2024

- GENERAL

| 1.1 | Contracting Authority, Legal and Postal Address | JSC RMG Group, Georgia, Bolnisi district, Kazreti village |
| --- | --- | --- |
| 1.2 | Property Location | JSC RMG Group, Georgia, Bolnisi district, Kazreti village |

- MAIN REQUIRENMENTS

It is required to design and commission program code and HMI screens for the filtration and thickening areas and integrate it to the existing DCS system on Madneuli copper concentrating plant for centralized control using Rockwell Automation software. Network diagram illustrates approximate organization of the network. Main PLC is installed in a cabinet PLC-FP-011 and control everything on both areas. This PLC must be integrated with the existing  DCS located in the flotation area, which is on a Rockwell Automation ControlLogix architecture.

The Main existing server for this DCS in the  concentrating plant consists of:

- HMI server for thin clients OWS4, OWS5, OWS6;

- Alarm and Events server;

- Historian server;

2 standalone HMIs will be used as backup machines when connection to the server fails.

Thickening area (Att.1 P&ID 596-ТХ2,3-P&ID_2024.06.26_(rev.6)) has 3 units with its own local PLCs (Siemens), Motor Control Center (MCC), Instruments, Valves, etc. The units based on Siemens PLCs are connected to the network via Ethernet IP/ profinet gateways. MCC is connected to the network, i.e. all motors are controlled via network connection using Ethernet IP protocol.

Filtration Area has a subsystem consisting of 10 press filters. This subsystem is supplied by the vendor. It is needed to integrate it to plant DCS.

This area also has instruments, valves and motors which are directly connected to the RIO cabinet. For information on this can be found in IO list.

The main requirements for the contractors scope is summarized below.

| 2.1 | Integration to DCS network of following units | - Thickener TTH01-ST02-TH-001 (Simatic DP CPU 1512SP F-1 PN); - Thickener TTH01-ST02-TH-002 (Simatic DP CPU 1512SP F-1 PN); - Flocculant dosing unit FLOC-CP-01 (Simatic S7-1200 CPU); - Press-Filter’s subsystem (RA 1756-L75 CPU); - Motor control centers: 25 VSDs and 15 DOLs (preliminary). This equipment is to be connected via ethernet communication; |
| --- | --- | --- |
| 2.2 | PLC Code type | Ladder Diagram, FBD, IEC 61131-3 |
| 2.3 | Number of IOs (Preliminary) | 1. Total AI: 96 2. Total AO: 24 3. Total DI: 224 4. Total DO: 224 |
| 2.4 | Network configuration |  |
| 2.5 | Elaboration of HMI screens related to Press Filter and Thickener areas using Rockwell FT View ver 11, which is utilized by PlantPax DCS, elaboration of PLC code using Rockwell Studio 5000 (version 32) in accordance with P&ID diagrams and control philosophy. |  |
| 2.6 | Participation in Commissioning of the equipment mentioned above |  |

- ADDITIONAL REQUIREMENTS

| 3.1 | Contractor must | - be Rockwell Automation expert; - have all licenses needed for work execution; - have an excessive experience in mining (particularly in thickening and filtration); - provide FAT before integration and commissioning. |
| --- | --- | --- |

- ATTACHMENT

- P&ID 596-ТХ2,3-P&ID_2024.06.26_(rev.6);

- Process Diagram of Filtration area;

- Network diagram;

- IO List for Thickener Area;

- HMI screen examples & libraries.