Technical service of high voltage stations of Tbilisi Mall.

Technical requirements

Contents:

•General Information------------------------------------- ----2

• Technical conclusion, --------------------------------- -----2

•Technical data ----------------------------------------------- -2

• Technical task--------------------------------------------- ---2

• Safety norms--------------------------------------------------3

• Evaluation------------------------------------------------ -----3

General Information

The building of the shopping center "Tbilisi Mall" is located at N 213 of Davit Agmashenebeli alley.

This building consists of three blocks (A, B and C blocks). There are 5 transformer stations on the 3rd and 4th floors of the building, which consist of 10 transformers and 5 high-voltage distribution devices, and 10 circuit breakers and 10 circuit breakers.

Technical conclusion

The above devices need to be serviced.

Technical data.

High voltage stations are equipped with transformers manufactured by Schneider company. Model (Schneider Trihal) with a capacity of 2000 kVA. Relay model (Schneider RM6) is equipped with a switch. Input voltage is 10KV.

Technical assignment

Technical services should include:

1) Measurement of the idle loss of transformers.

2) Measurement of the insulation parameters of the transformer.

3) Measuring the impedance of the transformer winding with constant current.

4) measurement of the transformation coefficient.

5) Measuring the polarity of the transformer.

High voltage circuit breaker and circuit breaker

1) Measurement of insulation characteristics.

2) Transient impedance measurement.

3) Testing the shutdown feature

4) high voltage test (40 kV).

5) High voltage testing of 10 kV cables feeding transformers.

6) Measurement of insulation characteristic of 10kV cables feeding transformers.

10 kV section

1) Measurement of insulation characteristics.

2) high voltage test. (40kV)

Standard works.

3) Testing contacts, cleaning devices.

Safety norms

All work that is carried out in the mall and in the surrounding area must meet the norms of labor and personal safety.

The person performing the work must have a qualification certificate, high-voltage e-mail. Equipment testing permit.

Before the start of work, the safety manager of the mall will inform the contractors about the safety norms.

The work will be allowed only if the document is signed by the working staff and the security manager of the mall.

assessment

All types of expenses must be specified in the submitted estimate.