Technical specifications for the purchase of equipment of JSC RMG GROUP

Electric single girder overhead crane with payload of 5 t.

Tbilisi

- Electric single girder overhead crane with payload of 5 t.

  - Purpose

The device is designed for loading and unloading water pumps in a building.

  - Requirements to the Supplier

The device is supplied by the manufacturer or official representative with experience in installation and maintenance, including at mining and processing plants.

  - Technical characteristics

| Payload, T | 5 |
| --- | --- |
| Number of cranes, pcs. | 1 |
| Span, m | 10,5 |
| Lifting height, m | Up to 10,0 |
| Crane movement speed, m/min | 1-40, standard 20 |
| hoist movement speed, m/min | 1-20, standard 20 |
| Hoist lifting speed, m/min | 1-8, standard 8 |
| Operating mode group according to ГОСТ 25546 state standard | 3К |
| voltage, v | 380 |
| Frequency, Hz | 50 |
| Lower limit of operating and non-operating temperature, °C | -20, standard operation |
| Working types | Installation, dismantling and repair of devices |
| crane operating temperature, °С | from -20 up to +40 |
| seismicity, МSK-64 | 9 |
| Degree of protection | IP-54 |

Additional options

| Radio control with F21 | yes |
| --- | --- |
| Radio controls without F21 connection | no |
| МР KATSU of high speed | no |
| Explosion-proof performance | no |
| Performance for cold climates (-40) | no |
| Fire safety performance | yes |
| У1 performance | no |

| Technical characteristics |  |
| --- | --- |
| Payload, t | 5 |
| Crane movement speed, m/min | 20 |
| Rated capacity, kW, no more | 2×0.4+8+0,37 |
| Crane rail type according to state standard ГОСТ 6368-52 | P75 |
| Supply voltage, v | 380 |
| Control circuit voltage, v | 24, 36, 42, 380 |
| The mass of the crane, T, no more | 2,62 |
| Mode of operation according to ISO 4301/1 | A3 |
| Climatic performance according to the state standard ГОСТ 15150-69 | У3 |
| Wheel load on the rail, calculated, kN, no more | 31.6 |

Engine-reducer

electric hoist

no more than 850

Electric single girder overhead crane with payload of 5 t.

  - Basic technical and technological requirements for material and technical resources

1.4.1.	The design and material characteristics of parts and systems must be suitable for climatic conditions and allow for rapid installation of wearing parts at the engine installation site.

1.4.2 The main blocks and units must operate on all-season hydraulic and transmission lubricants and oils.

1.4.3 The guaranteed service life of the electric motor and gearbox must be at least 10,000 hours.

1.4.4 Presentation of guaranteed time-limited work on key components and parts.

1.4.5 The crane must be equipped with a soft start and a hoist movement mechanism.

1.4.6 to be equipped with a brake on the hoist movement mechanism.

1.4.7 to be equipped with a load limiter.

1.4.8 hoist with disc brake on lifting mechanism.

- REQUIREMENTS FOR ELECTRICAL EQUIPMENT

Cable and wire products must be flexible (with multi-core conductors), with non-flammable insulation that does not lose its plasticity at temperatures from +45 to -20℃.

- Dimensional sizes

The supplier determines based on the internal overall dimensions chosen by him.

- WARRANTY PERIOD CONDITIONS

The warranty period is 12 months from the date of completion of acceptance tests or 24 months from the date of delivery to the Customer's warehouse. In the event of suspension of operation due to the fault of the Supplier, the warranty period is extended for the period of delay.

- საქონლის დაკომპლექტება:

| Number | name | Amount, unit |
| --- | --- | --- |
| 1 | Electric single girder overhead crane with apayload of 5 t | 1unit |
| 2 | A set of spare parts for the tool (SPTA) for repairs during a 24-month period of operation. | 1 set |

- List of required documents

The Supplier shall provide the Customer with technical documentation:

| № | Documentation | Date of submission** |
| --- | --- | --- |
| 1 | Dimensional drawing | 10 days after signing the agreement |
| 2 | Operation, technical service instruction | Together with the device |
| 3 | Spare parts catalogue (electronic version) | Together with the device |
| 4 | Drawings of short-life details | Together with the device |
| 5 | Device passport | 10 days after signing the agreement |
| 6 | electrical schemes | Together with the device |
| 7 | Certificates of conformity, origin | Together with the device |

** Drawings must be submitted electronically in PDF and DWG format

Documentation shall be provided in English, Russian languages.

- Packaging requirements

It is not regulated.

- Additional terms

Not applicable.

- Attracting sub-suppliers

It is allowed by agreement with the Customer.

- Terms of delivery

The product must be delivered under the following conditions: DDP urban-type settlement Kazreti station, Georgia, or FCA.