Technical Assignment for

Drilling machine for underground core drilling

CONTENTS

SECTION 1. GENERAL INFORMATION

Subsection 1.1 Name.

Subsection 1.2 Novelty information.

SECTION 2. FIELD OF APPLICATION

SECTION 3. OPERATING CONDITIONS

SECTION 4. TECHNICAL REQUIREMENTS

Subsection 4.1. Main parameters and dimensions.

Subsection 4.2. Main technical, economic and operational

Performance indicators

Subsection 4.3. Reliability requirements

Subsection 4.4 Requirements for materials and vehicle components

Subsection 4.5. Power requirements

Subsection 4.6. Documentation requirements

SECTION 5. REQUIREMENTS FOR THE SCOPE AND/OR DURATION OF

WARRANTIES

SECTION 6. MAINTAINABILITY REQUIREMENTS

SECTION 7. ENVIRONMENTAL REQUIREMENTS

SECTION 8. SAFETY REQUIREMENTS

SECTION 9. LIST OF ANNEXES

Translated with DeepL.com (free version)

1. GENERAL INFORMATION

1.1 Name

Self-propelled drilling exploration rig for core drilling

1.2 Information on novelty

The Supplier shall ensure that the equipment supplied is new, previously unused, serial model reflecting all recent modifications and not discontinued by the manufacturer at the time of delivery.

- SCOPE OF APPLICATION

The exploration drilling rig is designed for core drilling of boreholes up to 1000 meters deep, of various diameters. It is used in underground mine workings of medium and small cross-section and on the ground surface

- OPERATING CONDITIONS

For all macroclimatic regions on land, except for the climatic region with Antarctic cold climate.

4. TECHNICAL REQUIREMENTS

4.1 Main parameters and dimensions

Dimensions of the machine in folded state: Length - 4500 mm. Width - 2100 mm. Height - 1900 mm. Equipment type: Fully assembled on a single base Chassis type: Self-propelled on caterpillar with diesel engine Drilling module power unit: Electrically driven oil station Electric motor - 65-84 kW. 400V On-board electrical system 12V. Duplication of drilling operations control by remote operator console. Drilling parameters: Drilling depth (HQ drill pipe) - 400 meters. Drilling depth (NQ drill pipe) - 600 meters. Drilling depth (BQ drill pipe) - 1000 meters. Feed stroke - 3300 mm. Borehole inclination angle, deg. - 360º Feeding mode - fast, slow. Feeding force - 65kN Lifting force - 65kN Rotator speed range - 0 - 2200 rpm. Max torque on the rotator - 2000 N/m.

4.2 Technical, economic and operational indicators

Operating mode on demand.

4.3 Reliability requirements

Normative service life is not less than 4 years.

4.4 Requirements to materials and equipment components

- Protection of electrical equipment IP68 - Automatic fire extinguishing system. - Spare parts (set of filters for 250 drilling/h).

4.5 Power supply requirements

On-board network voltage 12 V. Motor voltage 400V

4.6 Documentation requirements

- Operation, maintenance and repair manual in electronic and paper format. - Operating, maintenance and repair instructions for the internal combustion engine on electronic and hard copy. - Spare parts catalog on electronic and hard copy.

- REQUIREMENTS TO THE SCOPE AND PERIOD OF WARRANTY PROVISION

The warranty service period is not less than 12 months. No limit to mot/hours.

6. MAINTAINABILITY REQUIREMENTS

Drilling rig parts must be able to withstand the forces and stresses to which they are subjected during active operation. Durability of the materials used should correspond to the envisaged operation, taking into account the appearance of danger associated with the phenomena of fatigue, aging, corrosion and wear. The operating manual (instructions) for the machine must specify the type and frequency of inspection and maintenance required to ensure safety. Parts, assemblies, units subject to wear and tear and the criteria for their evaluation and replacement must be specified.

7. ENVIRONMENTAL REQUIREMENTS

The exhaust gas content shall comply with the Euro -3 standard.

8. SAFETY REQUIREMENTS

The self-propelled geological exploration drilling rig for core drilling should be delivered in strict compliance with the “Safety rules for loading and unloading operations

CONSIDERED:

Chief mechanic Statsenko I.