Technical Assignment for the Procurement of Equipment by JSC RMG GROUP

Automatic Station for Preparation and Dosing of Flocculant

Tbilisi 2025

  - Automatic station for preparation and dosing of flocculant

  - Purpose of the Equipment

# The equipment is intended for the preparation and dosing of the flocculant solution.

  - Type, size, and design of the flocculant preparation station.

| 1 | Required flocculant (mixture) capacity, m³/h | 3 |
| --- | --- | --- |
| 2 | Concentration, % | From 0.05 |
| 3 | Concentration range of the standard solution, % | 0,1 – 0,16 |
| 4 | Mass concentration, kg/m³ | 10-16 |
| 5 | Material of the equipment | Stainless steel AISI 304 |
| 6 | Operating mode | Year-round operation, 24/7 |
| 7 | Spare parts kit | - Station can be operated in automatic, manual, and remote automatic modes; - Powder level sensor; - Overflow protection; - Adjustment of the standard flocculant flow rate. |
| 8 | Electrical supply: voltage, V / frequency, Hz | 400/50 |
| 9 | Degree of protection | IP65 |

# 1.3.  Spare parts kit

| Item No. | Name | Quantity (pcs.) |
| --- | --- | --- |
| 1 | Automatic flocculant preparation and dosing station | 2 pcs. |
| 2 | Spare parts kit for 24 months of operation | 1 pc. |

# 3. MAIN TECHNICAL AND TECHNOLOGICAL REQUIREMENTS FOR MTR

3.1 The characteristics of the design and materials of parts and systems must comply with the climatic conditions and allow for quick replacement of worn parts at the installation site of the station.

3.2 The main units and assemblies must operate on all-season hydraulic and transmission lubricants and oils.

3.3 The guaranteed service life of the station must be at least 10,000 hours.

3.4 Provision of guaranteed operating hours for the main units and assemblies of the station.

- REQUIREMENTS FOR ELECTRICAL EQUIPMENT

Cable and wire products must be flexible (with multi-core wires) with non-flammable insulation that retains its plasticity at temperatures from +40°C to -20°C.

- WARRANTY CONDITIONS

The warranty period is 12 months from the date of completion of the acceptance test or 24 months from the date of delivery to the customer's warehouse. In case of suspension of operation due to the supplier, the warranty period is extended for the downtime period.

- LIST OF REQUIRED DOCUMENTATION

The supplier provides the customer with the technical documentation:

| Item No. | Documentation | Delivery Time** |
| --- | --- | --- |
| 1 | Dimensional drawing | 10 days after signing the contract |
| 2 | Operation and maintenance manuals | Included with the equipment |
| 3 | Spare parts catalog (electronic version) | Included with the equipment |
| 4 | Drawings for rapid wear parts | Included with the equipment |
| 5 | Passport of the equipment | 10 days after signing the contract |
| 6 | Electrical diagrams | Included with the equipment |
| 7 | Certificates of conformity and origin | Included with the equipment |

** Drawings must be provided in electronic format (pdf and dwg).

Documentation is provided in English and Russian.