Flocculant Dosing Pumps

Tbilisi 2025

- Intended Use

The pump is designed for distributing water from the tailings thickener.

- SCOPE OF APPLICATION

Transfer and distribution of flocculant from flocculant stations SPP2.1/2.2.

- SUPPLIER REQUIREMENTS

The equipment supplier must be either the manufacturer or an authorized representative. The supplier must have experience in installation and maintenance, including at mining and mineral processing facilities.

- TECHNICAL SPECIFICATIONS

# 4.1.1 General Requirements for water distribution pumps

| № | Parameters | General Technical Requirements |
| --- | --- | --- |
| Type of Pump Equipment |  |  |
| Progressive cavity (screw) pumps |  |  |
| Operating Conditions: |  |  |
| 1 | Climate Version | Temperate |
| 2 | Ambient Operating Temperature, °C | -20…+40 |
| 3 | Corrosion Index (CIO) | 0,9 |
| 4 | Elevation Above Sea Level, m | 730,7 |
| 5 | Continuous Operation Time per Day, h | 24 |
| Pumped Medium Characteristics: |  |  |
| 6 | Maximum Flow Rate, m³/h | 3-4 |
| 7 | Minimum Flow Rate, m³/h | 1,1-1,12 |
| 8 | Specific Gravity of the Liquid, t/m³ | Flocculant |
| 9 | Type of Pumped Medium | +10…+25 |
| 10 | Temperature, ℃ | 7-7,5 |
| 11 | PH | - |
| 12 | Foaming Factor | Fine particles may be present |
| Electric Motor: |  |  |
| 13 | Protection Class (IP) | 65 |
| 14 | Voltage, V | 400 |
| 15 | Rated Frequency, Hz | 50 |
| 16 | Explosion-Proof Design | No |
| 17 | Variable Frequency Drive (VFD) | Yes |
| Installation Data: |  |  |
| 18 | Location | Indoors |
| 19 | Geodetic Head, m | (-2)…(-3) |
| 20 | Minimum Pump Head, m | 10 m (1 bar) |
| 21 | Nominal Pump Head, m | 12 m (1,2 bar) |
| 22 | Maximum Pump Head, m | 15 m (1,5 bar) |
| 23 | Water Source | Open tank |
| 24 | Water Level in Tank, m | 1 to 1.7 m |
| 25 | Number of Pumps | 2 |
| 26 | Parameter Controlled by VFD | Required |  |

  - Key Technical and Technological Requirements for the Pumping Unit (MPU)

    - The design features and materials of components and systems must be suitable for the specified climate conditions and allow for quick on-site replacement of wear parts.

    - Main blocks and assemblies must operate using all-season hydraulic and transmission lubricants and oils.

    - The guaranteed service life of the electric motor and gearbox must be no less than 10,000 operating hours.

    - The supplier must provide guaranteed operating time data for the main components and parts of the pump unit.

- ELECTRICAL EQUIPMENT REQUIREMENTS

Cable and wiring products must be flexible (multi-stranded) with non-flammable insulation that maintains flexibility within a temperature range of +45°C to –20°C.

- DIMENSIONS OF THE SCREW PUMP

Pump frame width (W): not more than 200 mm.

Pump frame length (L): not more than 720 mm.

Pump suction nozzle diameter: DN50, PN16, flanged connection.

Pump discharge nozzle diameter: DN50, PN16, flanged connection.

- WARRANTY CONDITIONS

The warranty period is 12 months from the date of completion of acceptance testing or 24 months from the date of delivery to the customer's warehouse, whichever occurs first. In the event of downtime caused by the supplier, the warranty period shall be extended by the duration of the downtime.

- PRODUCT COMPLETENESS:

| Item No. | Item Description | Quantity |
| --- | --- | --- |
| 1 | Pump with electric motor | 1 unit in operation + 1 unit in reserve |
| 2 | Spare parts kit for 24 months of operation | 1 set |

- REQUIRED list of DOCUMENTATION

The Supplier shall provide the Customer with the following technical documentation:

| Item No. | Documentation | Submission Deadline** |
| --- | --- | --- |
| 1 | Dimensional drawing | Within 10 days after contract signing |
| 2 | Operation and maintenance manuals | With the equipment |
|  | Spare parts catalogue (electronic version) | With the equipment |
| 4 | Drawings of wear parts | With the equipment |
| 5 | Equipment passport | Within 10 days after contract signing |
| 6 | Electrical diagrams | With the equipment |
| 7 | Certificates of conformity and origin | With the equipment |

** Drawings must be provided electronically (PDF and DWG).

Documentation shall be provided in both English and Russian.

- PACKAGING REQUIREMENTS

Not regulated.

- ADDITIONAL REQUIREMENTS

The pump must be a dosing pump.
The pump capacity shall range from 1 to 3 m³/h.
The pressure shall be no less than 10 to 15 m.

- SUBCONTRACTING

Permitted with the Customer's prior approval.

- DELIVERY TERMS

The equipment shall be delivered under DDP (Delivered Duty Paid) terms to the station of Kazreti, Georgia.