Emulsion ANFO

|  | COMPOSITION/INFORMATION ON INGREDIENTS |
| --- | --- |

| Substance | %(w/w) | CAS No | EC No | CPL- Classification | CLP-Classification |
| --- | --- | --- | --- | --- | --- |
| Amonium Nitrate | < 82 | 6484-52-2 | 229-347- 8 | O;R8 Xi;R36 | Ox. Solid 3-H272 Eye Irr. 2-H319 |
| Mineral Oil | < 2,6 | 8042-47-5 | 232-455- 8 | - | Asp. Tox.1-H304 |
| Surfactant | < 1,2 | - | - | Xn;R22 Xi;R36/R37/38 | Acute Tox. Inh. Cat.4-H332 Acute Tox. Skin Cat.4-H312 Acute Tox. Oral Cat.4-H302 Eye Irr. Cat.2-H319 |
| Fuel Oil | < 2,5 | 68334-30- 5 | 269-822- 7 | Car. Cat.3; R40 | Canc. Cat. 2-H351 |
| Sensitizing | < 0,2 | - | - | F;R11 | Flame.Solid.2,H228 |

|  | PHYSICAL AND CHEMICAL PROPERTIES |
| --- | --- |

# Appearance and Physical form 	: Emulsion gel  Colour 	 	 	 	: Light red Odour  	 	 	 	: N/A pH 	 	 	 	 	: N/A Melting Point 	 	 	: N/A Boiling Point  	 	 	: N/A

Date of Issue  	: 09.06.2016

## Rev. Number  	: 00 Rev. Date 	 	: - Page Number  	: 5/7

| Freezing Point | : N/A |
| --- | --- |
| Density | : 1,24 to 1,28 g / cm3 |
| Viscosity @ 40 0C | : 80.000-120.000 cP |
| Resolution | : Not Soluble |
| Decomposition Temperature | : Ammonium Nitrate: 210 ° C (410 ° F) |
| Vapor Pressure | : N/A |
| Oxidation Features | : Decomposition is a powerful oxidant |
| Explosion Feature | : Explosives, mass explosion damage |
| Explosion Data Explosion speed | : 4000-4500 m/s (125 mm cartridge exposed) |
| Static Explosion Data | : Not Sensitive Electrical Sensitivity |
| Explosion Data Impact | : Not Sensitive Sensitivity |