I&C Service Engineer, Ore Enrichment Plant Madneuli

Svintsov Stanislav

28 September 2025

TECHNICAL SPECIFICATION

For the procurement of an Ultrasonic Level Meter

| Item № | Parameter Name | Value |  |
| --- | --- | --- | --- |
| 1 | Customer | I&C Service, Ore Enrichment Plant Madneuli |  |
| 2 | Full name, position, phone, e-mail of the responsible executor / Customer’s representative | Svintsov S.P.., Engineer of I&C Service |  |
|  |  | Engineer of I&C Service |  |
|  |  | +995591957961 |  |
|  |  | SSvintsov@richmetalsgroup.com |  |
| 3 | Basis for procurement | Notice of unsuitability for use |  |
| 4 | Equipment name | Ultrasonic level meter |  |
| 5 | Quantity to be procured | 1 set |  |
| 6 | Technical characteristics, requirements | Measured medium characteristics: |  |
|  |  | Name | Slurry (Pulpa) |
|  |  | Temperature, Со | +30…+45 |
|  |  | Excess pressure, kPa | 100 to 101.3 kPa |
|  |  | Ambient characteristics: |  |
|  |  | Temperature, Со | +10…+35 |
|  |  | Humidity, % | 60…80 |
|  |  | Aggressiveness | yes |
|  |  | Dust presence | yes |
|  |  | Flowmeter characteristics: |  |
|  |  | Transducer type | Ultrasonic |
|  |  | Process connection | UNI flange 3"/DN80/80, PP max 2.5bar abs./ 36psia, suitable for NPS 3" Cl.150 / DN80 PN16 / 10K 80 |
|  |  | Power supply / Output signal | 2-/4-wire connection scheme (HART), PROFIBUS PA, FOUNDATION Fieldbus |
|  |  | Measured parameters | Liquids Bulk solids |
|  |  | Measuring range | Liquids: 10 m Bulk solids: 5 m |
|  |  | Ambient temperature range | -40 °C ... 80 °C |
|  |  | Protection rating | IP68 NEMA6P |
|  |  | Display | yes |
|  |  | Power supply / Communication | 2-/4-wire connection scheme (HART), PROFIBUS PA, FOUNDATION Fieldbus |
|  |  | Connection | 4 ... 20 mA HART PROFIBUS PA FOUNDATION Fieldbus |
|  |  | Error/inaccuracy | +/- 4 mm or +/- 0.2 % of the specified measuring range |
| 7 | Documentation | - Availability of operation and maintenance manual (documentation language – Russian, English). - Availability of type approval certificate for measuring instruments and calibration procedure – Not required. - Availability of verification certificate (initially verified) – Not required. |  |
| 8 | Warranty period | Not less than 1 year |  |
| 9 | Additional conditions | None |  |
| 10 | Other requirements | None |  |

Hide details

Approval - Non-hazardous area

Process Connection - UNI flange 3"/DN80/80, PPmax 2.5bar abs./ 36psia, suitable for NPS 3" Cl.150 / DN80 PN16 / 10K 80

Power Supply; Output - 2-wire; 4-20mA HART

Operation - 4-line display VU331, envelope curve display on site

Housing - F12 Alu, coated, IP68 NEMA6P

Cable Entry - Gland M20 (EEx d > thread M20)

Process Sealing Sensor / Flange - EPDM

Additional Option - Basic version

Marking - Tagging (TAG), see additional spec.