I&C Service Engineer, Ore Enrichment Plant Madneuli

Svintsov Stanislav

28 September 2025

TECHNICAL SPECIFICATION
For the procurement of a Digital Measuring Cable

| Item № | Parameter Name | Value |  |
| --- | --- | --- | --- |
| 1 | Customer | I&C Service, Ore Enrichment Plant Madneuli |  |
| 2 | Full name, position, phone, e-mail of the responsible executor / Customer’s representative | Svintsov S.P.., Engineer of I&C Service |  |
|  |  | Engineer of I&C Service |  |
|  |  | +995591957961 |  |
|  |  | SSvintsov@richmetalsgroup.com |  |
| 3 | Basis for procurement | Notice of unsuitability for use |  |
| 4 | Equipment name | Digital Measuring Cable |  |
| 5 | Quantity to be procured | 1 set |  |
| 6 | Technical characteristics, requirements | Measured medium characteristics: |  |
|  |  | Name | Slurry (Pulpa) |
|  |  | Temperature, Со | +30…+45 |
|  |  | Excess pressure, kPa | 100 to 101,3 kPa |
|  |  | Ambient characteristics |  |
|  |  | Temperature, Со | +10…+35 |
|  |  | Humidity, % | 60…80 |
|  |  | Aggressiveness | yes |
|  |  | Dust presence | yes |
|  |  | Measuring cable characteristics: |  |
|  |  | Transducer type | Potentiometric (Measuring cable for contactless, inductive, digital signal transmission) |
|  |  | Process connection | Inductive head – connector for digital sensors with Memosens technology and terminals for wires or M12 plug for connection to transmitter |
|  |  | Ambient temperature range | -25 to +135 °C (-13 to +277 °F) |
|  |  | Output signal | 4…20 mA |
|  |  | LCD indicator | No |
|  |  | Protection rating | Not less than IP68 |
|  |  | Explosion protection | ATEX / NEPSI II 1G Ex ia IIC T3/T4/T6 Ga IECEx ia IIC T3/T4/T6 Ga FM / CSA IS/NI CLI Div1&2 GP ABCD |
|  |  | Measuring range | High-impedance cable – not required |
|  |  | Cable length | 25 m |
|  |  | Wiring/Cable characteristics | 4-wire (Cable jacket: TPE, Diameter: 6.3 mm / 0.25 inch, Conductors: 2×2 twisted pairs) |
|  |  | Power supply | 24 V DC |
|  |  | Cable gland | required |
| 7 | Documentation | - Availability of operation and maintenance manual (documentation language – Russian, English). - Availability of type approval certificate for measuring instruments and calibration procedure – Not required. - Availability of verification certificate (initially verified) – Not required. |  |
| 8 | Warranty period | Not less than 1 year |  |
| 9 | Additional conditions | None |  |
| 10 | Other requirements | Compatibility with digital sensors using Memosens technology |  |

Hide details

Approval:

ATEX / NEPSI II 1G Ex ia IIC T3/T4/T6 Ga IECEx ia IIC T3/T4/T6 Ga FM / CSA IS/NI CLI Div1&2 GP ABCD

Cable Length:

25m

Cable Connection:

Wire terminals